YTRANS - file transfer CP/M <-> PC

Version 1.5, Pavel Zampach, 2021

Overview

--------

Data transfer between CP/M and PC can be done by various ways (moving floppy

disk, by memory card or by seriel port transfer).  For transfer one or a few

files is using of serial port via YMODEM convenient way. The transfer is slow

(about 16 K per minute with 9600 Bd) and probably will be used for small files

only. The YMODEM protocol is implemented in simple form, 1k/128-byte blocks

with checksum (no CRC). The YMODEM transfers filename and can transfer more

files in one session. And what do you need? CP/M computer (of course) and

serial connection to terminal program KiTTY or PUTTY running on Windows.

Installation

------------

* Copy files SY2.EXE and RY2.EXE into KiTTY/PUTTY folder. In session setup go

to Zmodem option and set the download folder. Next set "rz" as RY2.EXE and

"sz" as SY2.EXE, clear Options in both cases, or you can in "sz" set Options

to "-1k" for using only 128-byte blocks (see below).  Don't remember to save

new settings.

* Copy files YDOWN.COM and YUP.COM to CP/M machine

Files download (from CP/M to PC)

--------------

* Run

YDOWN [D[U]:]ufn{ [D[U]:]ufn}

(f.e. YDOWN B10:SOURCE.Z80 B3:MANUAL.DOC B2:MYTEXT.TXT) or

YDOWN [D[U]:]ufn.YFL

(f.e. YDOWN YTRANS.YFL)

and wait for start message (important!), filenames have to be unambiguous

(without wildcards "*", "?").

* Right click on PUTTY/KiTTY title bar and choose "Zmodem Receive".

* Wait for closing message.

* The downloaded files are placed into set download folder.

Files upload (from PC to CP/M)

------------

* Run

YUP [D[U]:] (f.e. YUP C3:)

and wait for start message (important!)

* Right click on PUTTY/KiTTY title bar and choose "Zmodem Upload".

* Choose one or more files for uploading (Shift-click, Ctrl-click).

* Wait for closing message,

* The uploaded files are stored into entered or default drive/user. Long

Windows filenames are converted into short (8.3) format.

Notes

-----

Files with extension YFL (Ytrans File List) are handled by special way. This

files are simple text files with one filename (path is optional) per one line.

Files from the list are tranferred as usual.

ZFILER (excellent file manager for ZCPR 3) can run YDOWN and YUP by macro

commands. Add next two lines into ZFILER.CMD (edit drives for YUP.COM and

YDOWN.COM):

U   $D$U:;CLS;A0:YUP;$H:

D , $D$U:;CLS;A0:YDOWN $F;$H:

and you can use macros <esc>U for upload and <esc>D for download. You can even

use group macro G<esc>D for download of tagged files (max. about 8 files).

CP/M programs (YDOWN and YUP) have full DU: support and both can be interrupted

before running PC counterpart by pressing any key.

Settings

--------

Following options can be set by *.CFG files (in Z-system) or by any hex

patcher.

* 1k block (010Bh) -  enable (0FFh) or disable (00h) using of 1k blocks in

YMODEM protocol. For some CP/M machines 128-byte blocks are needed.

* LED port (010Ch) - if your CP/M machine has LED on some I/O port, you can

set it for blinking as feedback in file transfer. Zero value disables

blinking.

* LED on (010Dh) - value sent to LED port for switch LED on.

* LED off (010Eh) - value sent to LED port for switch LED off.

* CPU speed (0124h) - poke here CPU speed in MHz (important for program

timing).

Experience

----------

Program was tested on two systems: RunCPM (CP/M emulator running on Arduino

Due, equivalent cca 9 MHz Z80 system) and SC126 (true CP/M 3, 18.4 MHz).

On RunCPM I have to use 128-byte blocks and speed 19200 Bd. SC126 works

reliably with 1k blocks and speed 115200 Bd (RTS/CTS enable) or 57600 Bd (w/o

handshaking).######################################