RunCMP/Arduino Due a time module DS3231

---------------------------------------

Pavel Zampach, 2020

Hardware

--------

Module DS3231 is connected to Arduino Due by standard way via I2C (pin 21 SCL,

pin 20 SDA). As VCC use 3.3 Volts only!

RunCPM system

-------------

In folder RUNCPM there are files RUNCMP.INO, CPM.H and DATETIME.H. Move them

into your RunCPM project (don't forget backup your original ones) and

recompile. Changes are marked by "#####". New BDOS functions ZSDOS compatible

are now available:

Number  Function name         Input Parameters         Returned Values

----------------------------------------------------------------------

39    Get fixed disk vector None                     HL=Fixed Disk Vector

45    Set error mode        Dummy in RunCPM

47    Get DMA address       None                     HL=Current DMA Address

48    Get DOS & version     None                     H=DOS type: "S"=ZSDOS

L=BCD Version (0x11)

98    Get time              DE=Address to Put Time   A=Time/Date Code

99    Set time              DE=Address of Time       A=Time/Date Code

100    Get flags             Dummy in RunCPM

101    Set flags             Dummy in RunCPM

102    Get file stamp        DE=Address of FCB        A=Time/Date Code,

Stamp in DMA Buffer

103    Set file stamp        DE=Address of FCB,       A=Time/Date Code

Stamp in DMA Buffer

Time/Date Codes:   A=01H if No error

A=FFH if error

Time format (6 bytes packed BCD):

TIME+0 - last 2 digits of year

(prefix 19 assumed for 78 to 99, else 20 assumed)

TIME+1 - month  [1..12]

TIME+2 - day    [1..31]

TIME+3 - hour   [0..23]

TIME+4 - minute [0..59]

TIME+5 - second [0..59]

File Stamp format (15 bytes packed BCD):

DMA+0  - Create field (first 5 bytes of time format)

DMA+5  - Access field (first 5 bytes of time format)

DMA+10 - Modify field (first 5 bytes of time format)

Utilities

---------

ZSDOS programs using system real-time clock (TD, ZCAL) are O.K. But programs

working with file date/time unfortunately always use direct access to

DataStamper module instead BDOS functions, so that they are unusable.

TD     - simple program for time display and settings (TD S)

ZCAL   - month calendar (patched for 21-th century :-), ZCAL ? for help

QDIR   - directory list with file size and modify time

Syntax:

QDIR [D[U]:][afn] [/[T|S|D][-]]

Sorting (default is file name):

T file type

S file size

D file date

Order (default is ascending)

- descending

PPIP   - ver. 1.9, preserving original file time

PPIPZ  - dtto with ZCPR3 support

TEX    - time scheduler for ZCPR3

ZCNFG  - program for configuration (TD, QDIR):

ZCNFG xxx.COM xxx.CFG